`                                                                    ns-3.42

#

include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/wifi-module.h"

#include "ns3/mobility-module.h"

#include "ns3/applications-module.h"

#include "ns3/flow-monitor-module.h"

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("TcpOver80211nExample");

int main (int argc, char *argv[])

{

double simTime = 10.0; // seconds

uint32_t payloadSize = 1024; // bytes

std::string tcpVariant = "TcpNewReno"; // defaut6 lt

CommandLine cmd;

cmd.AddValue ("tcpVariant", "TCP variant (TcpNewReno/TcpCubic/TcpBbr/TcpWestwoodPlus)", tcpVariant);

cmd.AddValue ("simTime", "Simulation time in seconds", simTime);

cmd.AddValue ("payloadSize", "Payload size in bytes", payloadSize);

cmd.Parse (argc, argv);

// Set TCP variant

if (tcpVariant == "TcpWestwood" || tcpVariant == "TcpWestwoodPlus")

tcpVariant = "TcpWestwoodPlus"; // correct class in ns-3.42

TypeId tcpTid;

NS_ABORT_MSG_UNLESS (TypeId::LookupByNameFailSafe ("ns3::" + tcpVariant, &tcpTid),

"TCP variant not available: " << tcpVariant);

Config::SetDefault ("ns3::TcpL4Protocol::SocketType", TypeIdValue (tcpTid));

Config::SetDefault ("ns3::TcpSocket::SegmentSize", UintegerValue (payloadSize));

NodeContainer wifiStaNodes;

wifiStaNodes.Create (1);

NodeContainer wifiApNode;

wifiApNode.Create (1);

YansWifiChannelHelper channel = YansWifiChannelHelper::Default ();

channel.SetPropagationDelay ("ns3::ConstantSpeedPropagationDelayModel");

channel.AddPropagationLoss ("ns3::FriisPropagationLossModel");

YansWifiPhyHelper phy;

phy.SetChannel (channel.Create ());

WifiHelper wifi;

wifi.SetStandard (WIFI_STANDARD_80211n);

wifi.SetRemoteStationManager ("ns3::MinstrelHtWifiManager");

WifiMacHelper mac;

Ssid ssid = Ssid ("ns3-80211n");

mac.SetType ("ns3::StaWifiMac",

"Ssid", SsidValue (ssid),

"ActiveProbing", BooleanValue (false));

NetDeviceContainer staDevice = wifi.Install (phy, mac, wifiStaNodes);

mac.SetType ("ns3::ApWifiMac",

"Ssid", SsidValue (ssid));

NetDeviceContainer apDevice = wifi.Install (phy, mac, wifiApNode);

MobilityHelper mobility;

mobility.SetMobilityModel ("ns3::ConstantPositionMobilityModel");

mobility.Install (wifiApNode);

mobility.Install (wifiStaNodes);

InternetStackHelper stack;

stack.Install (wifiApNode);

stack.Install (wifiStaNodes);

Ipv4AddressHelper address;

address.SetBase ("192.168.1.0", "255.255.255.0");

Ipv4InterfaceContainer staNodeInterface, apNodeInterface;

staNodeInterface = address.Assign (staDevice);

apNodeInterface = address.Assign (apDevice);

uint16_t port = 9;

Address sinkAddress (InetSocketAddress (apNodeInterface.GetAddress (0), port));

PacketSinkHelper packetSinkHelper ("ns3::TcpSocketFactory", sinkAddress);

ApplicationContainer sinkApps = packetSinkHelper.Install (wifiApNode.Get (0));

sinkApps.Start (Seconds (0.0));

sinkApps.Stop (Seconds (simTime + 1));

OnOffHelper server ("ns3::TcpSocketFactory", sinkAddress);

server.SetAttribute ("DataRate", DataRateValue (DataRate ("100Mbps")));

server.SetAttribute ("PacketSize", UintegerValue (payloadSize));

ApplicationContainer serverApps = server.Install (wifiStaNodes.Get (0));

serverApps.Start (Seconds (1.0));

serverApps.Stop (Seconds (simTime + 1));

FlowMonitorHelper flowmon;

Ptr<FlowMonitor> monitor = flowmon.InstallAll ();

Simulator::Stop (Seconds (simTime + 1));

Simulator::Run ();

// Calculate throughput

monitor->CheckForLostPackets ();

Ptr<Ipv4FlowClassifier> classifier = DynamicCast<Ipv4FlowClassifier> (flowmon.GetClassifier ());

std::map<FlowId, FlowMonitor::FlowStats> stats = monitor->GetFlowStats ();

for (auto const &flow : stats)

{

Ipv4FlowClassifier::FiveTuple t = classifier->FindFlow (flow.first);

if (t.destinationPort == port)

{

double throughput = (flow.second.rxBytes * 8.0) / (1e6 * simTime);

std::cout << "TCP Variant: " << tcpVariant

<< " Payload: " << payloadSize

<< " SimulationTime: " << simTime

<< " Throughput: " << throughput << " Mbps" << std::endl;

Simulator::Destroy ();

return 0;

}

run

./ns3 run "scratch/tcp802.cc --tcpVariant=TcpTahoe --simTime=10 --payloadSize=1024"

./ns3 run "scratch/tcp802.cc --tcpVariant=TcpNewReno --simTime=10 --payloadSize=1024"

./ns3 run "scratch/tcp802.cc --tcpVariant=TcpWestwoodPlus --simTime=10 --payloadSize=1024"

./ns3 run "scratch/tcp802.cc --tcpVariant=TcpCubic --simTime=10 --payloadSize=1024"

./ns3 run "scratch/tcp802.cc --tcpVariant=TcpLinuxReno --simTime=10 --payloadSize=1024"

change the payload size and run multiple times and take the values for gnuplotting.

plot-throughput.plt

# Set terminal and output file

set terminal png size 800,600

set output "throughput_plot.png"

# Set titles and labels

set title "Throughput vs Payload/Simulation Time"

set xlabel "X-axis (Payload Size in bytes / Simulation Time in s)"

set ylabel "Throughput (Mbps)"

set grid

# Plot the data

plot "results.dat" using 1:2 with linespoints title "TCP NewReno", \

"" using 1:3 with linespoints title "TCP Bic", \

"" using 1:4 with linespoints title "TCP Hybla", \

"" using 1:5 with linespoints title "TCP Westwood"

results.dat file format

payload - value1 - value2 - value3 - value4

1024 26.3 28.1 27.5 25.8

2048 30.0 32.5 31.0 29.7

Question:

plot the graph for “throughput” vs “simulation time”. (change the simTime while running the commands for different variants. Note the values and follow the ssame process as above)